MONPLUS

A Monitor for the Intel 8031 family

by Steve Kemplin

Introduction

MONPLUS is a monitor for the Intel 8031 family that allows the

user to:

1.   Read and modify internal registers (RAM,registers

and SFRs)

2.   Read and modify external RAM locations

3.   Run from a memory location

4.   Load RAM with an Intel Hex file from the host

computer

5.   Single-step through a program

6.   Call a user program as a subroutine. When a RET

instruction is encountered, control will return to

the monitor

7.   Dump memory data to a terminal

8.   Set up to 4 breakpoints

This monitor was designed for maximum usefulness while

requiring minimum memory.  Because of this, several items

within the monitor are hardware dependent, and unfortunately,

the monitor will probably have to be reassembled for your

memory environment.  The source code is included and is

written for use with the A51 assembler by PseudoCode

(registered trademark). The key values which may have to be

modified are all defined as equates in the first page of

source code.

The monitor's construction required some low-level routines

to be written which can also be used in user-downloaded

programs. These routines are accessible through a jump table.

This monitor uses some internal RAM locations (0x70 thru 0x7F

in the original source code) to "shadow" several registers and

for temporary storage.  In other words, the registers are

stored in these locations upon entry to the monitor, and are

restored prior to a user program being run or single-stepped.

During user program execution, the registers are not shadowed

of course.

As a general rule, any numbers which are typed into MONPLUS

are assumed to be hexadecimal.  The 0x notation is not

necessary. When the monitor is expecting a byte value, both

hex digits MUST be entered. When the monitor is expecting a

word value, all four hex digits MUST be entered. Before any

command is completely typed in, it may be aborted by typing

the ESCape key.

There are two "flavors" of the monitor in the archive. The

system I developed this for had very limited external memory,

but didn't make much use of the 8031's internal registers.

MONPLUS.ASM  therefore keeps its load buffer and breakpoint

table in 8031 internal memory. This means that all internal

RAM above 0x50 is used by the monitor. MONPLUSX.ASM keeps its

load buffer and breakpoint table in external RAM pointed to

by the constant RAMPAGE. MONPLUSX only uses internal RAM

locations 0x70-0x7F.

The monitor uses 3 bytes in external RAM (at 0x4FFC in the

original source code) to assemble a patch routine to allow

access to internal RAM. This RAM must be addressable as both

data and program memory. (Personally, I HATE Intel's way of

separating program and data spaces. I far prefer Zilog's

approach of bringing out a signal that indicates program/data.

If you need it, use it. If you don't need it, it doesn't cost

you an extra AND gate just to recombine program and data

memory.)

The monitor also uses external interrupt 0 in order to

implement single-stepping.  This means that any program

which disables this interrupt, either by messing with the

EA bit or with the EX0 bit will disable single-stepping.

Also, assuming the interrupt is still active, the monitor

may be entered by grounding INT0 or doing an LJMP 0x003.

The monitor uses P1 as a configuration port - specifically,

if P1.6 is grounded, it will vector to 0x2000 at power up.

Also, interrupts are vectored to various locations in the

0x2000 area.

This monitor is based in part on a monitor written and

copyrighted by Ron Stubbers. I've cleaned up some of the

internals, changed the user interface to one that I liked

better, and added TONS of new stuff.  Ron, thank you for

"contributing" most of the low level routines!

Interface

MONPLUS has been assembled to communicate with a terminal or

terminal-emulation program in ASCII at 1200 bits/sec, eight

data bits, no parity, and one stop bit. The monitor assumes

that it is talking to an ASCII terminal. Numbers are given in

hex, with A-F in upper case. For example, E5 would be

translated into the appropriate byte by the monitor.  Complete

address and data information must be given. Internal addresses

would thus be one byte and external addresses require two

bytes.  Examples of one and two byte addresses are 2A and E010

or 0000.

Operations

In the text which follows, any items in brackets [] are

optional. Any items in the command which appear in the text

description are enclosed in quotes. Any value not explicitly

stated as being a byte should be assumed to be a word value.

Breakpoint - B bpnum addr

B bpnum -

Allows the user to specify up to four breakpoints. "Bpnum" is

the breakpoint number and must be 1,2,3 or 4. The command

"B 1 2370" would set breakpoint 1 at address 2070. Code

execution could then be started by using a C, G, or S command.

The user program would then run until it hit location 2370.

Then the monitor would print "B=2370" indicating that the

breakpoint at 2370 had been reached. Breakpoints are NOT

automatically cleared when they are encountered as in many

other monitors. The command "B 1 -" would clear the

breakpoint.

In order for code to be breakpointed, it must reside in RAM,

and must be accessible as both program and data memory. The

reason for this is that the monitor "invisibly" replaces your

code with a call to a monitor routine. When the monitor is re-

entered, your code is restored.

If you put a breakpoint within 3 locations of an already

established breakpoint, strange things will happen. Be warned!

Call User Subroutine - C addr

This routine calls the user subroutine at "addr".  When a RET

instruction is encountered, the monitor is re-entered. This

allows a large block of code to be executed and control

returned to the monitor with a single command.

Dump Memory Contents - D addr count

This command dumps the contents of data memory to the terminal

in both hexadecimal and ASCII, starting at "addr" and

continuing for "count" bytes.  If the data and program spaces

overlap, it can also be used to dump program memory.  Dumps

are in the form of 16-byte rows and always begin on 16-byte

boundaries, so that if the starting address of 0013 were

given, 0010 would be the actual start address.  Full rows are

always output.  The format for the dump is as follows,

assuming 0000 as a start address, and 30 (hex) bytes:

0000 xx xx xx xx xx xx xx xx xx xx xx xx xx xx xx xx AAAAAAAAAAAAAAAA

0010 xx xx xx xx xx xx xx xx xx xx xx xx xx xx xx xx AAAAAAAAAAAAAAAA

0020 xx xx xx xx xx xx xx xx xx xx xx xx xx xx xx xx AAAAAAAAAAAAAAAA

The "xx"s and "A"s would, of course, be replaced by whatever

data is in the memory locations.

Fill memory - F addr count value

Fills memory with the byte "value", beginning at "addr" and

continuing for "count" bytes. No checking is done to assure

that the fill was successful.

Run from Address - G [addr]

This function allows the user to run a previously  loaded

program.  If "addr" is specified, execution begins from that

address; if "addr" is omitted, execution proceeds from the

address current when the monitor was last entered.  The

monitor may be re-entered by causing an INT0 either in

hardware or software (use LJMP 0x0003).

Read and Modify Internal RAM - I addr [value]

This function allows the user to read and modify the internal

RAM, including the SFRs.  When this command is  executed, the

current value of the internal RAM at "addr" is displayed.  If

the Return key is pressed at this point, control is returned

to the main monitor prompt.  If a new "value" is keyed in, it

will replace the old value and control will be returned to the

main prompt. No checking is don't to assure that the value

actually changed. Both "addr" and "value" are bytes.

Load Program RAM with Intel Hex File - L [addr]

An Intel Hex file can be downloaded to the system RAM  for

execution using this command.  The file is placed in  memory

as it was assembled.  If a program such as Procomm is used for

terminal emulation, the transfer is selected by uploading the

hex file using an ASCII transfer.  As each line of the file

is sent it will be displayed on the terminal, and at the end

of the transfer control will be returned to the monitor

program.  The file is transferred, but not executed.  It may

be executed using the G command.

If "addr" is not specified, the file will be loaded at the

addresses specified when the file was assembled. If "addr" is

specified, the code will be offset by "addr" bytes when it is

loaded. This is useful if you have a file that was assembled

to run at a location of 0x0000, but your PROM programmer is

located at an address of 0x2000. This command is not magic,

it does NOT reassemble your code, it just adds an offset to

each record as it is read.

No error checking is done.

Read and Modify External RAM - M addr [value]

This function permits the user to write a byte to any RAM

location within the 64K external data memory space.  When this

command is executed, the value stored at addr is  displayed.

If the space bar is pressed at this point, the value in the

next sequential memory location is displayed. If the "^" key

is pressed, the value in the next previous address is

displayed.  Any time the value is displayed, a new value may

be keyed in, and will replace the old value. Note that

attempting to write to ROM will not work, and will not result

in an error message.  The return key terminates this command.

Program EEPROM - P [addr]

This command is similar to the L command, but has much more

extensive error checking. After reading a line from the source

file, this command will respond with '?' if there was a

checksum error, '!' if the EEPROM didn't program correctly,

and '$' if everything was OK. It also puts in a 10 millisecond

delay so the EEPROM has time to program. Other than this, the

command is used just like the L command.

Register Examine and Modify - R regid [value]

Allows the examination and changing of a registers contents.

"Regid" is "A" for the accumulator, "B" for the B register,

"H" for the DPH register, "L" for the DPL register, "S" for

the PSW register or a number from 0-7 for R0 through R7. If

the byte "value" is specified, the register (actually its

stored image) is set to "value". If you want to look at a

register other than the ones specified above, you must use the

I command.

Single Step User Program - S [addr]

This function allows the user to single-step (execute 1

instruction and return to the monitor) a previously loaded

program.  If "addr" is specified, stepping begins at that

address; if "addr" is omitted, stepping proceeds from the

address current when the monitor was last entered.

Move memory - V addr1 addr2 count

The memory location at "addr1" is read and copied to "addr2".

Then, "addr1" and "addr2" are incremented. This process

continues until "count" bytes have been moved. "addr1",

"addr2", and "count" are all word values. No error checking

is done.

Jump Table

A jump table is provided so that the user can use several of

the low-level routines in the monitor. The basic idea is a bunch

of jumps at a known location in memory, with the monitor source

code set up so that if the monitor is reassembled, the jumps still

jump to the right place. Therefore, the user just does a LJMP or

a LCALL to a known location in order to read a character from the

serial port for example. These addresses are guaranteed to never

change. The addresses and the routines that they call are:

LOCATION  TITLE   DESCRIPTION

========  =====   ===========

0x30    INIT    ;Entry point to the monitor

;

0x33    DELAY   ;Pause for 1 msec times value in ACC

;

0x36    SERIN   ;Serial Input Primitive

;

0x39    GETC    ;Read char from serial port, echo char back

;to serial port. Convert lower case to upper

;

0x3C    PUTC    ;Send char in ACC to serial port

;

0x3F    PUTS    ;Sends text string to serial port,

;DPTR points to start, null ends

;

0x42    CRLF    ;Sends CR,LF to serial port

;

0x45    ASC2HEX ;Converts ASCII char in ACC to HEX equivalent,

;Returns value in lower nibble, upper nibble

;zeroes

;

0x48    HEX2ASC ;Convert lower nibble in ACC to ASCII char,

;returned in ACC

;

0x4B    RDHEX   ;Reads 2 char from serial port,

;converts to byte returned in ACC

;

0x4E    WRBYTE  ;Writes byte in ACC to serial port,

;as 2 ASCII char

;